# MLX5 Syslog Flood Workaround

Issue Summary, NVIDIA Fix Timeline, and RSyslog Workaround

BCM Infrastructure Team

March 12, 2026

# MLX5 Syslog Flood Workaround

## 1. Executive Summary

All DGX compute nodes are producing high-frequency kernel error messages from the Mellanox ConnectX (mlx5) driver approximately every 30 seconds. These messages flood /var/log/syslog and dmesg, consuming disk space and making log analysis difficult.

| Item | Detail |
| --- | --- |
| Severity | Medium — no functional impact to workloads |
| Affected | All DGX nodes (GPU and CPU) |
| Root Cause | mlx5 driver ACCESS_REG(0x805) register bug |
| Permanent Fix | DGX OS 7.4.0 (NVIDIA confirmed) |
| Interim Workaround | RSyslog filter (NVIDIA recommended) |
| Deployment Method | Ansible from BCM head node |

## 2. Issue Details

### Error Messages

The following kernel messages repeat every ~30 seconds on every node:

kernel: mlx5_core 0000:05:00.0: mlx5_cmd_out_err:839:(pid XXXXX):
  ACCESS_REG(0x805) op_mod(0x1) failed, status bad operation(0x2),
  syndrome (0x9a6171), err(-22)

kernel: mlx5_cmd_out_err: 16 callbacks suppressed

### Impact

- Disk Space: Syslog and dmesg logs grow rapidly, consuming storage

- Log Noise: Legitimate errors are buried in the noise

- Monitoring: Syslog-based alerting may trigger false positives

- No Functional Impact: The errors do not affect network performance, NCCL, or workload execution

### Observed On

- All DGX nodes across all availability zones

- First observed: March 5, 2026

- NVIDIA Support case opened and confirmed

## 3. Root Cause Analysis

The mlx5 (Mellanox ConnectX) network driver attempts to access hardware register 0x805 (ACCESS_REG command) repeatedly. This register access fails with:

- Status: bad operation (0x2)

- Syndrome: 0x9a6171

- Error code: -22 (EINVAL — invalid argument)

This is a known driver bug where the driver queries a register that is not supported in the current firmware/driver combination. The operation is harmless but generates kernel log entries on each retry.

## 4. Permanent Fix — DGX OS 7.4.0

NVIDIA has confirmed that this issue is resolved in DGX OS version 7.4.0.

- Reference: NVIDIA DGX OS 7 Resolved Issues — access-reg-command-failure-with-err-22

- NVIDIA Support Statement (March 9, 2026): “As you rightly noted, this issue was resolved in DGX OS version 7.4.0.”

Note: The OS upgrade path to 7.4.0 is not immediately available in our environment. The RSyslog workaround below provides an interim solution until the upgrade can be scheduled.

## 5. Recommended Workaround — RSyslog Filter

### What It Does

An RSyslog filter rule is deployed to each node that silences (drops) messages containing ACCESS_REG(0x805) before they reach /var/log/syslog. This is the exact workaround recommended by NVIDIA Support (engineer: Donghwan).

### Configuration File

File: /etc/rsyslog.d/10-mlx5-filter.conf

# MLX5 Kernel Noise Suppression (NVIDIA workaround)
# Issue: ACCESS_REG(0x805) op_mod(0x1) failed, err(-22)
# Fix: DGX OS 7.4.0
:msg, contains, "ACCESS_REG(0x805)" stop
:msg, contains, "mlx5_cmd_out_err" stop

### How RSyslog Filtering Works

Linux Kernel (printk)
        │
        ▼
systemd-journald (collects kernel messages)
        │
        ▼ (imjournal module)
rsyslogd
  ├─ Reads from journald via imjournal
  ├─ Applies filter rules from /etc/rsyslog.d/*.conf
  │   └─ 10-mlx5-filter.conf: DROP if msg contains "ACCESS_REG(0x805)"
  └─ Writes remaining messages to /var/log/syslog

Messages matching the filter are stopped before reaching any output action (file, remote, etc.).

### Alternative: Route to Separate File

If an audit trail is preferred over complete suppression, messages can be redirected to a separate file instead of dropped:

:msg, contains, "ACCESS_REG(0x805)" /var/log/kernel-noise.log
:msg, contains, "ACCESS_REG(0x805)" stop

This saves matching messages to /var/log/kernel-noise.log (with logrotate) while still keeping them out of /var/log/syslog.

## 6. Deployment Method — Ansible from BCM Head Node

### With Customer Consent

If approved, we will deploy the RSyslog filter via Ansible from the BCM head node:

# Deploy to all compute nodes
ansible-playbook -i inventories/<environment>/hosts.yml \
    playbooks/rsyslog-silence.yml

# Optional: dry-run first (no changes made)
ansible-playbook -i inventories/<environment>/hosts.yml \
    playbooks/rsyslog-silence.yml --check --diff

### What the Playbook Does

- Verifies RSyslog is installed on each target node

- Backs up existing RSyslog configuration

- Deploys the filter file to /etc/rsyslog.d/10-mlx5-filter.conf

- Validates the new config with rsyslogd -N1

- Restarts the RSyslog service

- Tests that messages are suppressed (via logger injection)

### CMSS Persistence (Survive Reimaging)

For nodes that may be reimaged, a BCM Configuration Overlay ensures the filter is automatically re-applied:

# Create CMSS overlay (run once on head node)
ansible-playbook -i inventories/<environment>/hosts.yml \
    playbooks/cmss-rsyslog-overlay.yml

## 7. Validation

After deployment, verification can be performed on any target node:

# 1. Check filter is deployed
cat /etc/rsyslog.d/10-mlx5-filter.conf

# 2. Check RSyslog is running
systemctl status rsyslog

# 3. Validate RSyslog config
sudo rsyslogd -N1

# 4. Test suppression via user-space logger
logger -p kern.info -t kernel "ACCESS_REG(0x805) TEST"
# Should NOT appear in: tail -f /var/log/syslog | grep ACCESS_REG

# 5. Test suppression via kernel message buffer
echo 'ACCESS_REG(0x805) TEST from /dev/kmsg' | sudo tee /dev/kmsg >/dev/null
# Should NOT appear in: tail -f /var/log/syslog | grep ACCESS_REG

## 8. Rollback Procedure

To remove the workaround (e.g., after upgrading to DGX OS 7.4.0):

# Via Ansible (recommended)
ansible-playbook -i inventories/<environment>/hosts.yml \
    playbooks/rsyslog-silence.yml --tags rollback

# Manual (per node)
sudo rm /etc/rsyslog.d/10-mlx5-filter.conf
sudo systemctl restart rsyslog

## Appendix: NVIDIA Support References

- NVIDIA Support Case: Noisy mlx5 kernel logs — ACCESS_REG(0x805) error

- NVIDIA Engineer: Donghwan (March 9, 2026)

- NVIDIA Guidance: Create /etc/rsyslog.d/10-mlx5-filter.conf with content :msg, contains, "ACCESS_REG(0x805)" stop, then systemctl restart rsyslog

- DGX OS 7.4.0 Release Notes: Resolved Issues — access-reg-command-failure-with-err-22